## Heat Sink Design Description:

## The project focuses on creating and analyzing a parametrically defined heat sink in FreeCAD using FEM thermal analysis and improving its thermal performance by modifying the fin geometry.

## The project involves the CAD design and thermal analysis of a compact heat sink on a 21 mm × 21 mm CPU. The heat sink consists of a 21 mm × 21 mm × 2 mm aluminum base with vertical cooling fins. Each fin is 2 mm thick, and the fin geometry is defined parametrically so that its dimensions can be modified during the thermal optimization process.

## The thermal performance of the heat sink will be evaluated using FEM with maximum temperature (Tmax) used as the primary performance metric. The fin geometry and spacing will then be modified to improve heat dissipation and reduce Tmax.

# 1. Requirements

### CAD requirements

- Heat sink base: 21 mm × 21 mm × 2 mm rectangular base.

- Cooling fins: Repeated vertical cooling fins arranged uniformly on the base.

- Material: Aluminum.

- Base dimensions: The overall 21 mm × 21 mm base dimensions must remain unchanged.

- Connectivity: All fins must remain physically connected to the heat sink base.

- Parametric design: Fin dimensions including fin height, thickness and spacing must be editable through FreeCAD parameters or features.

- Valid geometry: The final heat sink must consist of a single connected solid with no unintended gaps or disconnected bodies.

### Thermal requirements

- Heat Load: 5W

- Material: Aluminum

- Thermal conductivity: 205 W/(m·K)

- Applied heat flux: 11,338W/m²

- Ambient temperature: 25 °C

- Convection coefficient: 10 W/(m²·K)

- FEM mesh: Gmsh or FreeCAD FEM mesh

### Optimization requirement

- The primary performance metric: Maximum temperature, Tmax

- The optimized design should demonstrate a reduction in Tmax compared with the baseline design.

Task: Complete the thermal analysis and improve the heat-sink design

Starting from the provided FreeCAD project, complete the heat-sink thermal simulation.

- Create the parametric model of heat sink geometry with rectangular base and four fins

- Inspect the heat-sink geometry for errors.

- Complete the FEM analysis setup.

- Assign aluminum material properties.

- Generate an appropriate FEM mesh.

- Apply the specified heat flux to the heater surface.

- Apply convection to the exposed surfaces.

- Configure and run the steady-state thermal solver.

- Record the baseline maximum temperature.

- Modify the fin geometry according to the optimization requirement.

- Run the thermal simulation again.

- Confirm that the modified design has a lower maximum temperature than the baseline.

- Save the completed FreeCAD model.

# Verifier Criteria

### CAD verifier

Check:

- Heat-sink base exists.

- Fins are connected to the base.

- Overall base dimensions are correct.

- Fin geometry is present and editable.

- No unintended disconnected solids.

Pass: CAD geometry satisfies all required dimensions/features.

### Thermal verifier

Check:

- FEM Analysis object exists.

- Aluminum material is assigned.

- Thermal conductivity = 205 W/(m·K).

- Heat flux = 11,338W/m²

- Ambient temperature = 25 °C.

- Convection coefficient = 20 W/(m²·K).

- FEM mesh exists.

- Solver completes successfully.

- Temperature results are generated.

Pass: Complete thermal simulation produces a valid temperature field.

### Optimization verifier

Check:

- Fin geometry was modified.

- Base geometry remains within specified dimensions.

- Modified geometry remains connected/valid.

- Thermal simulation was rerun.

- Optimized Tmax is lower than baseline Tmax.

Pass condition:

Tmax,opt<Tmax,baseline